Simple AOS Socket

DRAFT SPECIFICATION

The TCP socket for the AOS allows external programs to communicate with the operating software. It will also notify programs of certain events (e.g. when a scan should be taken) and provide periodic (~ 2 – 10 seconds) machine status updates.

Socket information:

Address: 127.0.0.1

Default port: 10000

Connecting:

For a local client socket object named ‘MySocket’ the sequence of commands to connect to the AOS software might look like this:

void ClientClass::ConnectToAOS(TSocket * MySocket)

{//Set the socket parameters and connect

MySocket->Address = 127.0.0.1;	//Set the address to local host

MySocket->Port = 10000;		//Set the port to 10000

MySocket->Active = true;		//Send the connect request by making the port active

}

The AOS software will respond with the connected message. The client software should catch this message and respond with its identification. The identification message should be “UserName=xxxxx” where xxxx is the name of the program or optical system connecting to the AOS software (e.g. “Another optical system”). Note that no spaces are allowed. For example:

void ClientClass::MySocketConnect(TSocket *MySocket)

{//Connection established, inform AOS of the program name, add note to log

MySocket->SendText("UserName=MWL Optics”);

MySocket->SendText("<Experiment,Add Experimental Note,Connected>");

}

The first line of the example registers the name “MWL Optics” with the AOS software for use in notes, displays, etc. The second line will add the note “Connected” to the notes kept by the AOS software for the experiment (see command structure below). The note will appear in the records as “date time MWL Optics Connected”

Command structure:

The AOS Socket expects command requests formatted as either <System, Command> or <System, Command, Data>. The System defines what part of the operating system will get the command, Command is what operation is to be performed and Data is any information that a command requires. For example, sending the string “<Machine,Get Command List>” will return the list of commands available for the Machine system. Sending the string “<AOS, Write Status Bar, text>” will display the string in “text” at the bottom of the AOS program’s main form.

NOTE: In this early form, commands are exact. That is, the commands are case sensitive and with spaces as shown. I hope to make this less stringent in future releases.

Commands to AOS that set machine values (e.g. Set Desired Temperature) will activate the appropriate AOS input scheme (i.e. the Set Temperature form will be displayed) if the action is allowed. If the action is not allowed (e.g. a Stop/Pause command is sent when there is nothing to stop), no action is taken. Commands will return “True” if the appropriate input box can be opened, “False” if it cannot.

Data will be returned to the calling socket as strings in the form <System,Command,Data>, where System and Command echo the original command and Data is any value (if pertinent).

A few of the commands are done (below). The “Done” column tells whether the command has been implemented in the AOS server (“S”) or the example client (“C”) software. At present, there are still a large number of commands to be implemented. For example, those for the logical flags necessary to know what steps are in an experiment’s method, what the state of an ongoing experiment: is the program in a user-defined time delay before starting data acquisition, is it in a time interval between scans, how much time before scanning is to commence, etc. In addition, the AOS software will need to know some information about the optical system: 1) is the hardware present; 2) is the hardware ready (e.g. light sources and detectors warmed up); 3) is the hardware busy (i.e. gathering data).

Broadcast messages

The AOS software broadcasts messages to all clients. One of the commands allows clients to broadcast messages through the AOS software. Another command allows a client to obtain a list of all clients (addresses and names).

System name

Done  Command string		Message contents

AOS

None at present

Machine

S		Start Machine				Machine has been started. Pump down begins.

S		Stop Machine				Machine has been stopped. Rotor stopping, pumps off.

S		Settings Sent				A set of machine settings (rpm, temp, etc ) were sent

Experiment

S		Start Taking Data			Any delay and interval are done, start scanning

S		Next Method Step		The next step in the method will be executed

S		Using Method				Sent at the start of an experimental method

Data: the name of the method being used

S		Using Setup				Sent at the start of an experimental method

Data: the name of the method being used

S		Method Step				Sent at the start of a method step

Data: The rpm, temperature for the step

Optics

S		New Data File				File path where the data have been saved

Commands

The commands listed below provide access to the current experimental conditions, and allow external programs to change various conditions. More commands are being added as fast as I can program. I will update these data as they become available.

System name

Done  Command string		Return value or AOS action

AOS

SC	Get Command List			List of commands

SC	Get Systems					List of available systems

SC	Get AOS Version				AOS Version number

SC	Get DB Version				Machine DB version

S		Get Operating Mode			Testing, Beckman, Aviv

S		Get Connection List			String list of socket connections

SC	Write Status Bar				Send: string to appear on AOS status bar

Displays string

S		Get AOS Status				System status string

Machine

SC	Get Command List			List of commands

SC	Get Actual Temperature	XLA Temperature

SC	Get Desired Temperature	Temperature setting

SC	Set Desired Temperature	If successful, returns true, else false

SC	Get Actual Rotor Speed		XLA rotor speed

SC	Get Desired Rotor Speed	Desired speed

SC	Set Desired Rotor Speed 	If successful, returns true, else false

SC	Get Vacuum Status			Pump status, Value or -1 (not avail.)

SC	Get Machine Status			XLA status

SC	Get Omega Squared			w2t

S		Get Run Time					Seconds since rotor started spinning

SC	Get Run Time String		Time since started spinning “000:00” format

xx	S		Is Temperature Stable		Returns “True” if temperature is within set tolerance

S 		Is Stable State					Send: desired Rpm

Returns “True” if temperature, vacuum and the current rpm is within 100 rpm of the desired value

S		Is Rotor Spinning				Returns “True” if the rotor is spinning

S		Is Rotor Stopping				Returns “True” if the rotor is going to 0 rpm

S		Is Machine Started			Returns “True” if the XLA has been started

S		Is XLA Connected			Returns “True” if AOS & XLA communicating

Experiment

SC	Get Command List			List of commands

SC	Show Setup					Show setup

SC	Get Setup Name				Name of setup

S		Get Setup ID					Database entry number

SC	Show Method					Show method

SC	Get Method Name			Name of method

S		Get Method ID				Database entry number

S		Get Number Of Steps		Returns the number of steps in the method

S		Get Step Number				Returns the current step number (0 based)

S		Get Method Rpm				Returns the rpm for the current method step

S		Get Method Rpm Step		Returns the rpm for the step

Data: Step number (0 to last -1)

S		Get Method Temp			Returns the temperature for the current method step

S		Get Method Temp Step		Returns the temperature for the step

Data: Step number (0 to last-1)

S		Get Method Delay			Returns the delay in seconds for the current step

S		Get Method Delay Step		Returns the delay in seconds for step

Data: Step number (0 to last-1)

S		Get Method Interval			Returns the delay in seconds for the current step

S		Get Method Interval Step	Returns the delay in seconds for step

Data: Step number (0 to last-1)

S		Get Repeats					Returns the number of scans, or time, in current step

S		Get Repeats Step				Returns the number of scans, or time, for step

Data: Step number (0 to last-1)

NOTE:	Positive value is number of scans

Negative value is time (seconds)

SC	Get Experiment Name		Name of the experiment

S		Get Experiment ID			Database entry number

SC	Start/Resume					If OK, AOS presents the Start/Resume dialog

SC	Stop/Pause						If OK, AOS presents the Stop/Pause dialog

SC	Get Login						User login name

S		Set Login						Login user

SC	Get Data Path					Data file output path

SC	Add Experimental Note		Adds note to DB

xx	S		Is Experiment Started		Returns “True” if the experimental protocol started

S		Is Experiment Paused		Returns “True” if the protocol has been paused

S		Is Experiment Done			Returns “True” if the protocol has been completed

Optics

SC	Get Command List			List of commands

-		Get Available Optics		List of optics